LAB ASSIGNMENT 12

Problem Statement 1: Warp Divergence Analysis

Focus: Hardware Scheduling and SIMT Efficiency

Problem Description:

The objective of this study is to quantify the performance degradation caused by branch divergence. The student is required to design two kernels that perform identical total floating-point operations but differ in their thread-mapping logic.

- Divergent Scenario: Logic where even and odd threads within the same warp take different branches, forcing the warp to execute both paths serially.

- Warp-Aligned Scenario: Logic where branching is aligned with the 32-thread warp boundary, ensuring all threads in a warp remain synchronized.

Objective

Compare the execution times and Warp Execution Efficiency metrics using a profiler. Students must demonstrate how the physical grouping of threads influences the runtime of a kernel, even when the computational workload remains constant.

Problem Statement 2: Global Memory Throughput in Matrix Multiplication

Focus: Memory Coalescing and 2D Grid Mapping

Problem Description:

The task involves implementing a naive Matrix Multiplication kernel using a 2D grid of threads. Each thread (x, y) is responsible for computing a single dot product between a row of A and a column of B to produce one element of C.

Objective

Implement a functional 2D-mapped matrix multiplication kernel and calculate the Effective Memory Bandwidth achieved. Students will evaluate the impact of matrix dimensions on throughput